Purdue Pegboard Standard Operating Procedure

- Inform participants of the following information: “Now you will complete a test to see how quickly and accurately you can work with your hands. This test is split into four parts. Before you begin each part of the test, you will be told what to do and then have an opportunity to practice.”

- Participants and experimenters must wash their hands with soap and water

Part 1 – Right Hand

- Begin by saying and demonstrating: “Pick up one pin at a time with your right hand from the right-handed cup. Starting with the top hole, place each pin in the right-handed row”. Ask the participant if they would like to practice.

- When the participant understands the task, has returned any practice pins, and is ready to begin, say: “Please place your right hand on the edge of the table to the right of the board. When I say ‘Begin’, place as many pins as possible in the right-handed row, starting with the top hole. Work as rapidly as you can until I say ‘Stop’. Are you ready?”

- Prepare the timer, starting when you say “Begin”. At the end of exactly 30 seconds, say: “Stop”. Make sure the timer is out of view of the participant.

- Count the total number of pins inserted and record this as the Right Hand score. Leave the pins in the holes.

Part 2 – Left Hand

- Begin by saying (no demonstration needed): “Pick up one pin at a time with your left hand from the left-handed cup. Starting with the top hole, place each pin in the left-handed row.”

- Participants must start this part with their left hand to the left of the board. Starting with “Begin” and ending with “Stop” after 30 seconds, as before.

- Ask the participant to please return the pins into their respective cups.

Part 3 – Both Hands

- Begin by saying and demonstrating: “For this part of the test, you will use both hands at the same time. Pick up a pin from the right-handed cup with your right hand, and at the same time pick up a pin from the left-handed cup with your left hand. Then place the pins down the rows. Begin with the top hole of both rows. You may now practice.”

- Once the participant understands the task and has returned any practice pins, make sure they are in the start position (Both hands placed on the table to the both sides of the board)

- Once participants are in the start position, prepare the timer, starting when you say “Begin”. At the end of exactly 30 seconds, say: “Stop”

- Count the number of pairs of pins inserted (not the total number of pins) and record the Both Hands score.

- Ask the participant to please return the pins into their respective cups

Right + Left + Both

This score is not based on a separate test; simply add the scores for Right Hand, Left Hand, and Both Hands; this is the score you record for R + L + Both

Part 4 – Assembly

If the participant is right-handed, read out the red text, otherwise read out the blue text

- Begin by saying: “This final part of the test consists of assembling pins, collars, and washers”

Pins:				Collars:			Washers:

- Demonstrate the following operations while saying: “Pick up one pin from the right/left-handed cup with your right/left hand. While you are placing it in the top hole in the right/left-handed row, pick up a washer with your left/right hand. As soon as the pin has been placed, drop the washer over the pin. While the washer is being placed over the pin with your left/right hand, pick up a collar with your right/left hand. While the collar is being dropped over the pin, pick up another washer with your left/right hand and drop it over the collar. This completes the first ‘assembly’, consisting of a pin, a washer, a collar, and a washer.

While the final washer for the first assembly is being placed with your left/right hand, start the second assembly immediately by picking up another pin with your right/left hand. Place it in the next hole; drop a washer over it with your left/right hand, and so on, completing another assembly. Both hands should be always operating. Now take a moment to try a few practice assemblies.”

- The participant should be allowed to make four or five complete assemblies before the test is begun to make certain they fully understand the ‘alternating’ procedure. The participant must keep both hands moving at the same time; if they fail to do this, the experimenter should give further instructions. After practice, ask the participant to please return the pins, collars and washers to their respective cups and “please place both hands on the edge of the table to either side of the board”

- Prepare the timer, starting when you say “Begin.” At the end of exactly 60 seconds, say: “Stop.”

- Count the number of parts assembled and record the Assembly score. Since there are four parts in each assembly, if the participant made eight complete assemblies, the score is 8 multiplied by 4 (parts), or 32. Beyond completed assemblies, if there are additional parts properly placed at the end of the minute, they are also added to the Assembly score.